import socket

import numpy as np

import time

from adafruit_pca9685 import PCA9685

import board

import busio

print("5 DOF REAL ROBOT (SOCKET MODE)")

# -------------------------------

# PCA9685 SETUP

# -------------------------------

i2c = busio.I2C(board.SCL, board.SDA)

pca = PCA9685(i2c)

pca.frequency = 50

servo_channels = [0, 1, 2, 3, 4]

# -------------------------------

# ANGLE -> DUTY (FOR PCA9685)

# -------------------------------

def angle_to_duty(angle):

pulse_us = 500 + (angle / 180.0) * 2000

duty = int(pulse_us * 65535 / 20000)      # convert to 16-bit

return duty

# -------------------------------

# SEND TO SERVOS (FIXED)

# -------------------------------

def send_angles(angles):

angles = np.clip(angles, 0, 180)  # safety

for i in range(5):

duty = angle_to_duty(float(angles[i]))

pca.channels[servo_channels[i]].duty_cycle = duty

# -------------------------------

# SKILL TRANSFER (7 -> 5 DOF)

# -------------------------------

def map_7dof_to_5dof(q7):

j1 = q7[0]  # base (invert)

j2 = -(1.2*q7[1] + 0.8*q7[2])  # shoulder

j3 = q7[3]  # elbow

j4 = -(0.3*q7[4] + 0.2*q7[5])  # wrist

j5 = q7[6]  # end

return np.array([j1, j2, j3, j4, j5])

# -------------------------------

# SOCKET SERVER

# -------------------------------

HOST = "0.0.0.0"

PORT = 5005

server = socket.socket(socket.AF_INET, socket.SOCK_STREAM)

server.bind((HOST, PORT))

server.listen(1)

print("Waiting for Gazebo connection...")

conn, addr = server.accept()

print("Connected from:", addr)

# -------------------------------

# RECEIVE LOOP (FINAL)

# -------------------------------

buffer = ""

while True:

chunk = conn.recv(1024).decode()

if not chunk:

continue

buffer += chunk

while "\n" in buffer:

line, buffer = buffer.split("\n", 1)

try:

# Parse 7 DOF

q7 = np.array([float(x) for x in line.split(",")])

##   print("Received q7:", q7)

# Mapping

q5 = map_7dof_to_5dof(q7)

# Radians -> Degrees

angles = np.degrees(q5)

# SHIFT (center servo)

angles = angles + 90

# CLAMP (0-180 safe)

angles = np.clip(angles, 0, 180)

##   print("Angles:", angles)

# Send to robot

send_angles(angles)

except Exception as e:

print("Parse error:", e)